PICACHO ADJUSTING

- DATA CHECKING: Hold K0 + power ON, and get in the data checking.

| 0. TOTAL PROFIT SCORE | 1. TOTAL BET SCORE FOR EACH GAME |
| --- | --- |
| 2. TOTAL WINNING SCORE OF EACH GAME | 3. TOTAL DOUBLE UP SCORE |
| 4. TOTAL WINNING DOUBLE UP SCORE |  |

- K0: DATA CHECKING. K1: TESTING. K1+K3: COUNTING

- RESET: K0+K1 Power ON, reset

- ERROR 01: coin slid error

02: motor error, meter error

03: password error

76: key out error

| DIP SW | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | OFF |  |  |  |  |  |  |  |
| BET 1:1 |  | OFF |  |  |  |  |  |  |
| BET 1:5 |  | ON |  |  |  |  |  |  |
| COIN 1:1 |  |  | OFF | OFF |  |  |  |  |
| COIN 1:5 |  |  | ON | OFF |  |  |  |  |
| COIN 1:10 |  |  | OFF | ON |  |  |  |  |
| COIN 1:50 |  |  | ON | ON |  |  |  |  |
| DOUBLE UP 60% |  |  |  |  | OFF | OFF |  |  |
| DOUBLE UP 75% |  |  |  |  | OFF | ON |  |  |
| DOUBLE UP 85% |  |  |  |  | ON | OFF |  |  |
| DOUBLE UP 90% |  |  |  |  | ON | ON |  |  |
| RATIO 65% |  |  |  |  |  |  | OFF | OFF |
| RATIO 75% |  |  |  |  |  |  | OFF | ON |
| RATIO 85% |  |  |  |  |  |  | ON | OFF |
| RATIO 90% |  |  |  |  |  |  | ON | ON |

|  | J1 | J2 | J3 | J4 |
| --- | --- | --- | --- | --- |
| 1 |  | +5V | METER IN | SMALL MARIO KEYBOARD |
| 2 | GND | +5V | METER OUT |  |
| 3 | GND | GND | +12V |  |
| 4 | KEY IN | GND |  |  |
| 5 | GND | SSR | +12V |  |
| 6 | HP | +12V | +12V |  |
| 7 | KEY OUT |  | SP+ |  |
| 8 |  |  | GND |  |
| 9 |  |  | COIN+ |  |
| 10 | +5 |  | GND |  |