# XTIMER JAMMA

# Manual Pulsador XBOX

| XBOX Button | COMPONENT SIDE |  | SOLDER SIDE |  | XBOX Button |
| --- | --- | --- | --- | --- | --- |
|  | 1P TABLE |  | 2P TABLE |  |  |
| LT1 | 1P KEY7 | 28 | 28 | 2P KEY7 | LT1 |
| White | 1P KEY6 | 27 | 27 | 2P KEY6 | White |
| Black | 1P KEY5 | 26 | 26 | 2P KEY5 | Black |
| Y | 1P KEY4 | 25 | 25 | 2P KEY4 | Y |
| X | 1P KEY3 | 24 | 24 | 2P KEY3 | X |
| B | 1P KEY2 | 23 | 23 | 2P KEY2 | B |
| A | 1P KEY1 | 22 | 22 | 2P KEY1 | A |
| RIGHT | 1P RIGHT | 21 | 21 | 2P RIGHT | RIGHT |
| LEFT | 1P LEFT | 20 | 20 | 2P LEFT | LEFT |
| DOWN | 1P DOWN | 19 | 19 | 2P DOWN | DOWN |
| UP | 1P UP | 18 | 18 | 2P UP | UP |
| START | 1P START | 17 | 17 | 2P START | START |
|  | 1P COIN | 16 | 16 | 2P COIN |  |
| RT1 | 1P KEY8 | 15 | 15 | 2P KEY8 | RT1 |
|  | VIDEO GND | 14 | 14 | MENU KEY |  |
|  | BLUE | 13 | 13 | SYN |  |
|  | RED | 12 | 12 | GREEN |  |
| BACK | 1P KEY9 | 11 | 11 | 2P KEY9 | BACK |
|  | SPEAKER | 10 | 10 | SPEAKER |  |
|  |  | 9 | 9 |  |  |
|  |  | 8 | 8 |  |  |
|  |  | 7 | 7 |  |  |
|  | + 12V | 6 | 6 | + 12V |  |
|  |  | 5 | 5 |  |  |
|  |  | 4 | 4 |  |  |
|  |  | 3 | 3 |  |  |
|  | GND | 2 | 2 | GND |  |
|  | GND | 1 | 1 | GND |  |

NOTE: XTIMER NEEDN＊T POWER SUPPLIER WHEN CONNECT TO TV AS BELOW

| PLAY TIME PER COIN | DIP-SWITCH 1-5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |
| Test Mode | ---- | ---- | ---- | ---- | ---- |
| 1 MINS | ON | ---- | ---- | ---- | ---- |
| 2 MINS | ---- | ON | ---- | ---- | ---- |
| 3 MINS | ON | ON | ---- | ---- | ---- |
| 4 MINS | ---- | ---- | ON | ---- | ---- |
| 5 MINS | ON | ---- | ON | ---- | ---- |
| 6 MINS | ---- | ON | ON | ---- | ---- |
| 7 MINS | ON | ON | ON | ---- | ---- |
| 8 MINS | ---- | ---- | ---- | ON | ---- |
| 9 MINS | ON | ---- | ---- | ON | ---- |
| 10 MINS | ---- | ON | ---- | ON | ---- |
| 11 MINS | ON | ON | ---- | ON | ---- |
| 12 MINS | ---- | ---- | ON | ON | ---- |
| 13 MINS | ON | ---- | ON | ON | ---- |
| 14 MINS | ---- | ON | ON | ON | ---- |
| 15 MINS | ON | ON | ON | ON | ---- |
| 16 MINS | ---- | ---- | ---- | ---- | ON |
| 17 MINS | ON | ---- | ---- | ---- | ON |
| 18 MINS | ---- | ON | ---- | ---- | ON |
| 19 MINS | ON | ON | ---- | ---- | ON |
| 20 MINS | ---- | ---- | ON | ---- | ON |
| 21 MINS | ON | ---- | ON | ---- | ON |
| 22 MINS | ---- | ON | ON | ---- | ON |
| 23 MINS | ON | ON | ON | ---- | ON |
| 24 MINS | ---- | ---- | ---- | ON | ON |
| 25 MINS | ON | ---- | ---- | ON | ON |
| 26 MINS | ---- | ON | ---- | ON | ON |
| 27 MINS | ON | ON | ---- | ON | ON |
| 28 MINS | ---- | ---- | ON | ON | ON |
| 29 MINS | ON | ---- | ON | ON | ON |
| 30 MINS | ---- | ON | ON | ON | ON |
| FREE PLAY | ON | ON | ON | ON | ON |

* Test Mode = XBOX hand controller (Use to copy CD to hard disk)
DIP-SWITCH 6

| With Demo Sound | ON |
| --- | --- |
| Without Demo Sound | ---- |

DIP-SWITCH 7

| Return To Menu After Time Out | ON |
| --- | --- |
| Don＊t Return To Menu After Time Out | ---- |

*Some XBOX games can＊t return to menu on new version XBOX console
You have to put DIP-Switch 7 OFF and reboot XBOX console to enter menu
DIP-SWITCH 8

| 1 Coin Slot (1P / 2P COM) | ON |
| --- | --- |
| 2 Coin Slot (1P / 2P Independent) | ---- |

*The 16th pin of JAMMA on solder side will be disabled when DIP-Switch 8 put ON